# Kristal Farms — Northern Village Precipitation Source Data

**Working source-data file:** annual rainfall and total precipitation, by meteorological station or regional proxy

**Prepared:** 2026-05-02

**Purpose:** screening support for the Kristal Farms village inventory

---

## 1. Use and limitations

- This is a **screening document only**. It records the precipitation data currently used in the village inventory and separates local station values from regional proxy assignments.

- **Rainfall** means liquid precipitation only, in millimetres per year.

- **Total precipitation** means rainfall plus snow water-equivalent, as reported by climate normals or by the cited source.

- Hydropower screening should **not** rank sites by rainfall alone. The next screen must include watershed area, usable head/drop, winter low-flow, snowpack timing, permafrost, lake storage, port/fibre access, and FPIC/community readiness.

- **Hopedale is flagged as a working value**. Verify it directly in ECCC bulk CSV before design or investment use.

---

## 2. Source station / proxy data

| Station / proxy | Region | Source basis | Rainfall mm/yr | Total precip. mm/yr | Data type | Confidence |

|---|---|---|---:|---:|---|---|

| Nain | NL / Nunatsiavut | ECCC Climate Normals 1991–2020 bulk data; station NAIN | 468.8 | 870.9 | Local station | High |

| Hopedale | NL / Nunatsiavut | ECCC-derived station/weatherbox value; source not fully re-opened in this run | 378.1 | 822.0 | Local station / working | Medium |

| Inukjuak Upper Air | QC / Nunavik | ECCC-derived 1991–2020 weatherbox for Inukjuak | 264.6 | 459.9 | Local station | High |

| Kuujjuaq | QC / Nunavik | ECCC Climate Normals 1991–2020 bulk data; station KUUJJUAQ | 343.2 | 578.2 | Local station | High |

| Kuujjuarapik | QC / Nunavik | ECCC Climate Normals 1991–2020 bulk data; station KUUJJUARAPIK | 436.1 | 664.0 | Local station | High |

| Salluit / northern Hudson Strait | QC / Nunavik | CEN Salluit Research Station / regional estimate | ~150 | ~300 | Research-station / regional estimate | Medium-low |

---

## 3. Per-village assignment table

| Village / community | Assigned station or proxy | Rainfall mm/yr | Total precip. mm/yr | Basis | Confidence | Notes |

|---|---|---:|---:|---|---|---|

| Nain | Nain station | 468.8 | 870.9 | Measured / near-measured | High | Best precipitation support among core candidates. |

| Hopedale | Hopedale station | 378.1 | 822.0 | Measured / working | Medium | Re-check ECCC bulk CSV before engineering use. |

| Makkovik | Labrador coast proxy: Hopedale–Nain range | ~378–469 | ~822–871 | Proxy | Medium-low | Needs local station or gridded extraction. |

| Postville | Hopedale proxy | ~378 | ~822 | Proxy | Medium-low | Needs local station or gridded extraction. |

| Rigolet | Labrador coast proxy: Hopedale–Nain range | ~378–469 | ~822–871 | Proxy | Medium-low | Needs local station or gridded extraction. |

| Inukjuak | Inukjuak Upper Air station | 264.6 | 459.9 | Measured / near-measured | High | Hydro already proven locally through the Innavik reference case. |

| Puvirnituq | Central Hudson Bay proxy: Inukjuak | ~265 | ~460 | Proxy | Medium-low | Screen watershed area, head and winter low-flow. |

| Akulivik | Central Hudson Bay proxy: Inukjuak | ~265 | ~460 | Proxy | Medium-low | Screen watershed area, head and winter low-flow. |

| Kuujjuarapik | Kuujjuarapik station | 436.1 | 664.0 | Measured / near-measured | High | Keep mainly as context, not an active target. |

| Umiujaq | Southern Hudson Bay proxy: Kuujjuarapik | ~436 | ~664 | Proxy | Medium-low | Check protected-area constraints before hydrology work. |

| Chisasibi | Southern Hudson / James Bay proxy: Kuujjuarapik | ~436 | ~664 | Proxy | Low | Boundary/community reference. |

| Kuujjuaq | Kuujjuaq station | 343.2 | 578.2 | Measured / near-measured | High | Useful regional benchmark. |

| Tasiujaq | Ungava Bay proxy: Kuujjuaq | ~343 | ~578 | Proxy | Medium-low | Needs local basin hydrology. |

| Aupaluk | Ungava Bay proxy: Kuujjuaq | ~343 | ~578 | Proxy | Medium-low | Needs local basin hydrology. |

| Kangiqsualujjuaq | Ungava Bay proxy: Kuujjuaq | ~343 | ~578 | Proxy | Medium-low | Large-river potential is too large/remote for preferred model. |

| Kangirsuk | Transition proxy: Kuujjuaq to northern Hudson Strait | ~300–343 | ~450–578 | Proxy range | Low | Requires local station/gridded extraction. |

| Quaqtaq | Northern Hudson Strait proxy: Salluit-type | ~150–220 | ~300–335 | Proxy range | Low | Dry-region screen; verify winter low-flow and head. |

| Kangiqsujuaq | Northern Hudson Strait proxy: Salluit-type | ~150–220 | ~300–335 | Proxy range | Low | Dry-region screen; verify winter low-flow and head. |

| Salluit | Salluit research-station estimate | ~150 | ~300 | Regional estimate | Medium-low | Source says annual precipitation about 300 mm, 50% snow. |

| Ivujivik | Northern Hudson Strait proxy: Salluit-type | ~150–220 | ~300–335 | Proxy range | Low | Dry-region screen; verify winter low-flow and head. |

| Naniwa Sipi / Qikirtajuaq area | Ungava Bay geographic proxy: Kuujjuaq | ~343 | ~578 | Geographic proxy | Low | Geographic anchor only; not a confirmed village target. |

---

## 4. Source references

| ID | Source | What it supports | URL |

|---|---|---|---|

| S1 | ECCC Canadian Climate Normals 1991–2020 | Method/source framework: Climate Normals are based on composite stations with at least 15 years of data between 1991 and 2020; bulk CSV downloads are available. | <https://climate.weather.gc.ca/climate_normals/> |

| S2 | ECCC bulk-data result: NAIN | Annual rainfall 468.8 mm; annual total precipitation 870.9 mm. | <https://climate.weather.gc.ca/climate_normals/bulk_data_e.html?climateID=&lang=e&prov=NL&stnID=71000000&stnname=NAIN&yr=1991> |

| S3 | Inukjuak weatherbox / ECCC-derived 1991–2020 normals | Annual rainfall 264.6 mm; annual total precipitation 459.9 mm. | <https://en.wikipedia.org/wiki/Template:Inukjuak_weatherbox> |

| S4 | ECCC bulk-data result: KUUJJUAQ | Annual rainfall 343.2 mm; annual total precipitation 578.2 mm. | <https://climate.weather.gc.ca/climate_normals/bulk_data_e.html?climateID=&lang=e&prov=QC&stnID=105000000&stnname=KUUJJUAQ&yr=1991> |

| S5 | ECCC bulk-data result: KUUJJUARAPIK | Annual rainfall 436.1 mm; annual total precipitation 664.0 mm. | <https://climate.weather.gc.ca/climate_normals/bulk_data_e.html?climate_id=7103536&lang=e&prov=QC&stnID=106000000&stnname=KUUJJUARAPIK&yr=1991> |

| S6 | CEN Salluit Research Station / INTERACT GIS | Mean annual precipitation around 300 mm; about 50% snow, used to infer roughly 150 mm rainfall as a screening estimate. | <https://www.interact-gis.org/Home/Station/64> |

| S7 | KRG Nunavik RMMP 2021–2027 | Regional context: northern Nunavik total annual precipitation around 300 mm; southern Nunavik around 700 mm. | <https://www.krg.ca/iu/assets/environment/RMMP-Nunavik-21-27-Draft.pdf> |

| S8 | Hopedale source status | Working rainfall 378.1 mm and total precipitation 822.0 mm retained from previous dataset. Verify directly in ECCC bulk CSV before engineering use. | <https://en.wikipedia.org/wiki/Hopedale,_Newfoundland_and_Labrador> |

---

## 5. Recommended next data upgrade

1. Download the ECCC 1991–2020 Climate Normals bulk CSV for Newfoundland and Labrador and Quebec; extract annual rainfall and annual total precipitation for all available stations near the listed villages.

2. For villages without a local station, extract 1991–2020 gridded precipitation at exact village coordinates and at each candidate watershed centroid.

3. Replace single-point proxies with watershed precipitation statistics: mean annual precipitation, seasonal snow water-equivalent, winter low-flow proxy and interannual variability.

4. Keep this table as an audit trail: measured station, proxy used, confidence and date of last verification.

---

## 6. Data quality flags

| Flag | Meaning |

|---|---|

| High | Direct local station or near-local station data from a climate-normal-style source. |

| Medium | Working station value retained from prior dataset but not fully re-verified in the current run. |

| Medium-low | Regional station or research-station proxy; useful for screening only. |

| Low | Broad regional proxy or geographic assignment; requires local station, gridded extraction, or watershed hydrology work. |